COURSE CODE: CS372IA

USN

RV COLLEGE OF ENGINEERING®

(Autonomous Institution affiliated to VTU, Belagavi)

VII Semester B. E

Model Question paper

DEPARTMENT COMPUTER SCIENCE AND ENGINEERING

PARALLEL ARCHITECTURE AND GPU PROGRAMMING

Time: 03 Hours						          	          Maximum Marks: 100

Instructions to candidates:

- Answer all questions from Part A. Part A questions should be answered in first three pages of the answer book only.

- Answer FIVE full questions from Part B. In Part B question number 2 is compulsory. Answer any one full question from 3 and 4, 5 and 6, 7 and 8, 9 and 10.

|  |  | PART-A |  |
| --- | --- | --- | --- |
| 1 | 1.1 | Define a warp | 02 |
|  | 1.2 | List the orderings that sequential consistency needs to maintain. | 02 |
|  | 1.3 | Define loop-carried dependence with example | 02 |
|  | 1.4 | Give the command to compile Open ACC program. | 01 |
|  | 1.5 | Consider the multiplication of two 5 X 5 matrices using a vector processor. How many product terms are there in each inner product? How many multiply-add operations are needed to calculate the product matrix? | 02 |
|  | 1.6 | The number and size of tasks into which a problem is decomposed determines the ____________________ of the decomposition. | 01 |
|  | 1.7 | The VMIPS instruction for gather and scatter are ______ and ________ | 02 |
|  | 1.8 | The process of dividing a computation into smaller parts, some or all of which may potentially be executed in parallel, is called____ | 01 |
|  | 1.9 | ______ a computation is a technique of breaking a single large operation into smaller, independent pieces. | 01 |
|  | 1.10 | A _____directive can trigger the device initialization, device memory allocations, and data transfers without explicitly specifying respective operations. | 01 |
|  | 1.11 | Give an example for WAW and WAR hazards. | 02 |
|  | 1.12 | A_____ occurs when multiple threads race to perform some operation on a shared data item. | 01 |
|  | 1.13 | Sequential consistency is simple to understand but difficult to implement efficiently give reason | 02 |
| PART-B |  |  |  |
| 2 | a | List and explain different principles of computer architecture design. | 08 |
|  | b | Assume a disk subsystem with the following components and MTTF: ■ 10 disks, each rated at 1,000,000-hour MTTF ■ 1 ATA controller, 500,000-hour MTTF ■ 1 power supply, 200,000-hour MTTF ■ 1 fan, 200,000-hour MTTF ■ 1 ATA cable, 1,000,000-hour MTTF Using the simplifying assumptions that the lifetimes are exponentially distributed and that failures are independent, compute the MTTF of the system as a whole. | 08 |
| 3 | a | Explain cache coherence protocol generally used in UMA processors with detail state transition diagram based on request generated from CPU. | 10 |
|  | b | Assume that words x1 and x2 are in the same cache block, which is in the unchaced state in for both processors P1 and P2. Assuming the following sequence of events, identify each miss as a true sharing miss, a false sharing miss, or a hit etc. TIME; P1; P2. Read X1. Read X1. Write X2. Write X2. Write X1. Read X2. Read X1. | 06 |
|  |  | OR |  |
| 4 | a | Explain how coherence problem is solved by directory based write invalidate protocol with a neat state transition diagram for the directory structure. | 10 |
|  | b | Demonstrate how memory location XX21 can be locked using atomic test and set instruction. | 06 |
| 5 | a | Illustrate the basic structure and functionalities of vector architecture, VMIPS. | 08 |
|  | b | Discuss the architectural difference of CPU and GPU. Explain how GPU exploits data level parallelism. | 08 |
|  |  | OR |  |
| 6 | a | Briefly describe SIMD instruction set extensions for multimedia | 08 |
|  | b | Show how the following code sequence lays out in convoys, assuming a single copy of each vector functional unit: LV V1,Rx ;load vector X MULVS.D V2,V1,F0 ;vector-scalar multiply LV V3,Ry ;load vector Y ADDVV.D V4,V2,V3 ;add two vectors SV V4,Ry ;store the sum How many chimes will this vector sequence take? How many cycles per FLOP (floating-point operation) are needed, ignoring vector instruction issue overhead? | 08 |
| 7 | a | For the task graphs given in Figure, determine the following: 1. Maximum degree of concurrency. 2. Critical path length. 3. Maximum achievable speedup over one process assuming that an arbitrarily large number of processes is available. 4. The minimum number of processes needed to obtain the maximum possible speedup. | 08 |
|  | b | Discuss the use of Exploratory and Speculative decomposition techniques with suitable example | 08 |
|  |  | OR |  |
| 8 | a | Illustrate any 2 Parallel algorithms models in detail with respect to message passing interface | 08 |
|  | b | Write an MPI program that computes the value of PI using Monto Carlo algorithm | 08 |
| 9 | a | With an example code explain the reduction clause in OpenAcc | 08 |
|  | b | Explain how computation offloading is used in OpenACC | 08 |
|  |  | OR |  |
| 10 | a | Categorize the different clauses used in OpenACC | 08 |
|  | b | Exploring the differences between the two main types of OpenACC directives you can use to parallelize loops - kernels and parallel using a real code example | 08 |